7AG       A.O.P. Exam      7/4/27

Theory

- Define

- Kelvin

- Electro-static induction

- Henry

- Impedance

2 	How would you adjust a low power transmitter to obtain maximum efficiency in connection with the following:

a) Accuracy of frequency

b) Power radiated from antenna

c) Power input to anode cct.

- State advantages and disadvantages of following Xmitting ccts

1 Hartley  2 Meisner  (sic)  3 Colpitts

Which do you prefer and why?

- Explain the theory of the super-het cct. & give reasons for its design

- Discuss merits and demerits of foll. Systems of Xmission

- 3rd harmonic  b) Fundamental

- State what you know about Piezo-electric effect

- In establishing a low power radiophone installation Heising system how would you decide the following

- Relation between modulation and oscillator power

- System of plate supply

- System of filament supply

Give diagram where necessary

7AG       A.O.P. Exam      7/4/27

- Define the classes of stations that xperimenters are permitted to communicate with

- Give an example of a message of not more that 20 words. Show the preamble, text and signature and include the correct position of the necessary break signs.

- If it is necessary to communicate with a station whose call you do not know, what sigs would you make.

- State fully the classes of traffic that experimental stations are permitted to handle.

- Indicate the meaning of the following characters

_ … _          _._ _._        _.._ .        …_._

- What is the correct form of acknowledging the receipt of a message?

- What action do you consider necessary for a licenced experimenter to take in order to co-operate with the Department in the inspection of his station?